Poor Running at Low Speed:

Spark weak:

Battery voltage low 
Spark plug dirty, broken, or maladjusted 
Spark plug cap or high tension wiring trouble 
Spark plug cap shorted or not in good contact 
Spark plug incorrect 
igniter trouble 
Pickup coil trouble 
Ignition coil trouble

Fuel/air mixture incorrect:

Pilot screw maladjusted 
Pilot jet, or air passage clogged 
Air bleed pipe, bleed holes clogged 
Pilot passage clogged 
Air cleaner clogged, poorly sealed or missing 
Starter plunger stuck open 
Fuel level in carburetor float chamber too high or too low 
Fuel tank air vent obstructed 
Carburetor holder loose 
Surge tank duct loose

Compression low:

Spark plug loose 
Cylinder head not sufficiently tightened down 
Cylinder, piston worn 
Piston ring bad (worn, weak, broken, or sticking) 
Piston ring/land clearance excessive 
Cylinder head gasket damaged 
Cylinder head warped 
Engine not sufficiently warmed up after lash adjuster installation 
Valve spring broken or weak 
Valve not seating properly (valve bent, worn, or carbon accumulation on the seating surface) 
Hydraulic lash adjuster damaged (worn, seizure, or spring broken) 
Hydraulic lash adjuster oil passage clogged

Backfiring when decelerating:

Vacuum switch valve broken 
Air suction valve trouble 
Coasting enricher trouble

Other:

Igniter trouble 
Carburetors not synchronizing 
Carburetor vacuum piston doesn't slide smoothly 
Engine oil viscosity too high 
Drive train trouble 
Final gear case oil viscosity too high 
Brake dragging 
Air suction valve trouble 
Vacuum switch valve trouble