Motorcycle Transmission Problems

By Mark Shively

Slipping out of Gear:

Trying to mate slower spinning sliding gears with very high spinning fixed gears is hard on the transmission. If the transmission is slipping out of gears, the gear's dogs and cogs are rounded (worn) and must be replaced.

Race Shifting:

Shifting gears at or near redline pre-load the shift lever with your toe (raise the lever). When you want to shift quickly twist the throttle closed and then full-throttle again. Don't use the clutch. The transmission will shift into the next gear. This is a racing technique known as "pre-loading the shifter.” It should shift as quickly as you can turn the throttle. It's not the best thing for the clutch and transmission, but racing MC's get complete rebuilds frequently.

Inside the Engine:

When you step on the shift lever, the shift linkage turns the shift drum slightly and causes shifting forks to move. Shifting forks mate with shift drum via dowels and grooves. The shift forks move sliding gears to engage the gear's dogs with its mating gear's cogs.

When a gear is “forced” to engage another gear, as if trying to shift from Neutral to the next gear, expensive damages may result.   This causes premature transmission and clutch parts wear and failure.

Hauling in Neutral:

Tie-down straps secure our motorcycles and ATV’s well when used properly during transportation. We tend to use a variety of materials and things to secure motorcycles and ATV’s when transporting.

When the motorcycle/ATV is not properly secured, it may move during transport. If the transmission is placed in gear, shift forks flex under the load and weight of the motorcycle as it moves. Same goes with the clutch and drive chain. These parts are easily damaged. Place the transmission into Neutral and properly secure motorcycle with quality tie-down straps.